XSO Convection Batch Super Oven                                        www.BestBatchOven.com


How to upgrade the firmware for XSO-SuperOven

Receive the latest firmware version from Bokar International by e-mail.

1. Copy the file firmware “XSO-SuperOven_firmware.zip” and unpack it to one folder        (All files must be in the  same folder). 

            Password for unpacking: Oven
2. Connect XSO SuperOven (RS-485 Connector) to a PC (USB) using X485-USB converter. 

NOTE:   X485-USB Converter needs installed driver for proper operation.

            If the converter is connected to the PC for the first time,  install the X485-USB 
            driver.   

3. Press and hold the enter button on the Oven

4. Turn Power switch to ON position.

5. Release “Enter” button. The oven display should show text “Bootloader mode” 

6. Run program Flasher.exe on the PC (double click on the flasher icon)

7. Flasher should find the serial port where the oven is connected.

8. Flasher will ask “Do you want to start the update for your device?” 

9. Select “Yes” to confirm and press “enter”.

10. After upgrade is finished, you should get the message “Write successful. Would you like to run application?”

11. Press “ENTER”

12. Application will start.  The display in the oven should show X-KAR logo, the name, the serial number and the firmware version number.

13. Flasher window will automatically close.

Bokar International    650 Scranton Pocono Hwy, Covington Twp., PA 18444   USA  

                                                                www.bokar.com

